Programmable Interval Timer (PIT)

ASIC Diploma

Najah Now

Introduction

About PIT

The Programmable Interval Timer Module, PIT, is a simple timer to generate a periodic signal for a microcontroller system. This signal may be used for a variety of purposes such as triggering the start of an Analog to Digital or Digital to Analog conversion, as a periodic system interrupt, or to synchronize the start of various other hardware processes.

The PIT core has 4 primary blocks:

- WISHBONE Interface

- Control Registers

- Prescale Counter

- Main Counter

PIT structure

Clocks

We have only a master clock for the whole system named wb_clk_i with max frequency = 200MHz so its period = 1/200*10^6 = 5ns.

Tools

Synopsys tools were used to implement this project such as:

Design Compiler – ICC2 Compiler – Prime Time

==	==	==	==	==	==	==	==	==	==	==	==	==

Synthesis

Synthesis was performed using Design Compiler tool.

We used saed32nm library to implement the project.

- Design libraries

link and target libraries were set with min and max libraries to perform the correct timing analysis.

We used:

Min corner: ff1p16vn40c for Hold timing analysis

Max corner: ss0p75v125c for Setup timing analysis

- Design Constraints

- Clock definition with clock period = 5ns

- Input delay    = 0.2 * clock_period

- Output delay = 0.4 * clock_period

- wire load model  “35000”

- operating conditions

“ff1p16vn40c” for min corner and “ss0p75v125c” for max corner

- input transition (using “AND2X1_HVT” from max corner to derive the input)

- output load = 1 ff

- compile stage

We compiled the design using compile_ultra command and some app options to optimize the design.

- Generating design files

- SDC file

- Design netlist

Synthesis results summery

| Report | Result |
| --- | --- |
| Setup Time | 0.02 (MET) |
| Hold Time | 0.05 (MET) |
| Area | 1311.83 |
| Power | 6.8310e+03 uW |

- reports

setup Report

Hold Report

Area Report

Power Report

QOR Report

PnR

PnR stage was done using ICC2 tool from Synopsys.

In this stage we performed all PnR stages such as: floorplanning, Powerplanning, Placement, CTS, Routing.

First stage was data setup to set the libraries, ndm, technology file, read parasitics tluplus files, read design netlist and source design constraints.

- Floorplanning

- In this stage we defined routing tracks (M1 to M9) directions and placed the pins arround the rectangular core.

- Utilization was 40%

- Powerplanning

We used M8 metal layer for vertical mesh and  M9 for the horizontal mesh.

- Placement

MMMC file was sourced to set the needed files and corners for timing checking.

- CTS

- Clock network was built on metal layers M5, M6, M7.

- Trget skew was setted by 0.2.

- Clock latency was computed by the tool.

- Routing

PnR results summery

| Report | Result |
| --- | --- |
| Setup Time | 0.00 (MET) |
| Hold Time | -0.44 (VIOLATED) |
| Area | 2264.17 |
| Power | 2.59e+08 pW |

QOR Report

Timing 							Area

DRC

Congestion Report

Prime Time Session

First, we set up the libraries and important files such as: design netlist, parasitics file(.spef), constraints file.

Then we updated the timing, power, and noise info to start checking timing, power, and noise.

After this we performed an ECO fixing Session to fix timing and DRC violations.

After the ECO fixing, we get these results on PrimeTime tool

Results summery

| Report | Result |
| --- | --- |
| Setup Time | 0.265 (MET) |
| Hold Time | 0.002 (MET) |

Costraints

Setup

Hold